Objective: To ensure the safe operation of scissor lifts in the workplace, preventing accidents, and promoting a secure working environment.

Responsibilities:

- Operator Responsibilities:

  - Only trained and authorised personnel are allowed to operate scissor lifts.

  - Conduct a pre-operation inspection before each use.

  - Adhere to all safety guidelines and procedures outlined in this SOP.

  - Communicate any concerns or malfunctions to supervisors promptly.

  - Wear appropriate personal protective equipment (PPE) during operation.

- Supervisor Responsibilities:

  - Ensure that only trained and authorised personnel operate scissor lifts.

  - Verify that pre-operation inspections are conducted and documented.

  - Monitor compliance with safety procedures and address any violations promptly.

  - Provide necessary training and retraining as needed.

Pre-Operation Inspection:

- Check Controls:

  - Ensure that all controls are functional and respond appropriately.

  - Test emergency controls, including the stop button.

  - Address any issues with controls before operation.

- Check Hydraulic System:

  - Inspect for any visible leaks in the hydraulic system.

  - Ensure that the lift responds smoothly to control inputs.

  - Report any hydraulic issues immediately to maintenance.

- Inspect Platform and Guardrails:

  - Verify that the platform and guardrails are secure and undamaged.

  - Confirm that entry gates are functioning correctly.

  - Address any damage promptly to prevent accidents.

- Check Emergency Lowering System:

  - Test the emergency lowering system to ensure it works effectively.

  - Ensure all personnel are aware of the emergency lowering procedure.

- Inspect Tires and Wheels:

  - Check for any visible damage to tires and ensure proper inflation.

  - Verify that all wheels are in good condition.

  - Report any tire or wheel issues for maintenance.

Hazards and Risks:

- Fall Hazards:

  - Operating at heights poses a risk of falls. Always use guardrails and fall protection devices.

  - Stay within the designated work platform and never lean over the guardrails.

- Crushing Hazards:

  - Pinching and crushing hazards exist during scissor lift movement. Keep hands, feet, and clothing clear of moving parts.

  - Ensure no personnel are beneath the scissor lift during operation.

- Electrocution Risks:

  - Be aware of overhead power lines and maintain a safe distance.

  - Do not operate the scissor lift in adverse weather conditions like thunderstorms.

Dos and Don'ts:

Dos:

- Do wear appropriate PPE, including a hard hat and a safety harness.

- Do follow the manufacturer's guidelines for operation.

- Do communicate effectively with other personnel in the vicinity.

- Do use caution when driving the scissor lift, especially on uneven surfaces.

Don'ts:

- Don't exceed the designated load capacity of the scissor lift.

- Don't use the scissor lift as a crane or for any purpose other than its intended use.

- Don't override safety devices or attempt to disable any safety features.

- Don't operate the scissor lift if there are unresolved safety concerns or malfunctions.

Post-Operation Procedures:

- Shutdown:

  - Return the scissor lift to the designated storage area.

  - Turn off the power and secure the unit.

- Post-Operation Inspection:

  - Conduct a post-operation inspection and document any issues or concerns.

  - Report any defects or malfunctions to the supervisor.

  - Ensure proper storage and charging of the scissor lift.

Training: All personnel operating scissor lifts must undergo comprehensive training, including both theoretical and practical components.

Documentation: All inspections, training records, and incident reports must be documented and filed for future reference.

Review and Revision: This SOP will be reviewed periodically and revised as necessary to ensure its continued effectiveness.

I, ___________________hereby acknowledge receipt and understanding of the Safe Operating Procedure (SOP) document provided by. I affirm my commitment to adhere to the outlined procedures meticulously, ensuring the safety of myself and my colleagues. Furthermore, I acknowledge that I have undergone thorough training on the SOP to equip myself with the necessary knowledge and skills to execute tasks safely and efficiently. By signing this acknowledgment, I affirm my dedication to maintaining a secure work environment and contributing to the overall safety culture of the business.

Workers Signature:

Date: