| Main Contractor |  | Main Contractor Contact Person |  |
| --- | --- | --- | --- |
| Project Location |  | Contact person Contact Number |  |
| Date |  | Person approving JSA |  |
| Job Description: Drilling holes through cladding, installing brackets |  |  |  |
| PPE Requirements: Hard hat, Safety Glasses, High Visibility vest, Safety Boots |  |  |  |

| Worker Name Team Members who were consulted in the development of this JSA. Sign in below to confirm understanding of the JSA | Worker Signature | Worker Position | Date |
| --- | --- | --- | --- |

| Level | Descriptor | Examples |
| --- | --- | --- |
| 1 | Insignificant | No injury, no financial loss |
| 2 | Minor | First aid treatment, on-site release immediately contained, medium financial loss |
| 3 | Moderate | Medical treatment required, on-site release contained with outside assistance, high financial loss |
| 4 | Major | Extensive injuries, loss of production capability, off-site release with no detrimental effects, major financial loss |
| 5 | Catastrophic | Death, toxic release off-site with detrimental effect, huge financial loss |

Step1: Determine the consequence

Step 2: determine the Likelihood

| Level | Descriptor | Examples |
| --- | --- | --- |
| A | V Likely | Is expected to occur in most circumstances |
| B | Likely | Will probably occur in most circumstances |
| C | Moderate | Might occur at some time |
| D | Unlikely | Could occur at some time |
| E | Rare | May occur only in exceptional circumstances |

Step 3: Use the Matrix to determine the risk

|  | Consequence |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Likelihood | Insignificant 1 | Minor 2 | Moderate 3 | Major 4 | Catastrophic 5 |
| Very Likely A | High | High | Urgent | Urgent | Urgent |
| Likely B | Medium | High | High | Urgent | Urgent |
| Moderate C | Low | Medium | High | Urgent | Urgent |
| Unlikely D | Low | Low | Medium | High | Urgent |
| Rare E | Low | Low | Medium | High | High |

| Risk Level | Recommended Action |
| --- | --- |
| Urgent | Act Immediately The proposed task or process activity must not proceed. Steps must be taken to lower the risk level to as low as reasonably practicable using the hierarchy of risk controls |
| High | Act Within The proposed activity can only proceed, provided that: - The risk level has been reduced to as low as reasonably practicable using the hierarchy of risk controls - The risk controls must include those identified in legislation, Australian Standards, codes of practice - The risk assessment has been reviewed and approved - A Safe Working Procedure (SWP) has been prepared - The manager must review and document the effectiveness of the implemented risk controls. |
| Medium | Act Within The proposed task or process can proceed, provided that: - The risk level has been reduced to as low as practicable using the hierarchy of risk controls - The risk assessment has been reviewed and approved - A Safe Working Procedure (SWP) has been prepared. |
| Low | Act Within Managed by local documented routine procedures which must include application of the hierarchy of controls. |

| Step No. | Job Step | hazards | Risk | Controls | Residual Risk | Action |
| --- | --- | --- | --- | --- | --- | --- |
|  | List the tasks required to perform the activity | list the hazards that could cause injury | H | List the control measures required to eliminate or minimise the risk of injury |  | Name of person applying the controls |
| 1 | Visual/Manual inspection of the ladder before use. | - Cuts/Punctures/Scrapes Muscle Pull/Strain | H | - Check for loose or bent ladder rungs. - Check the “feet” or base of the ladder for cracks, bends, and missing parts. - Check screws, bolts, rivets, and other metal parts to make sure they are not loose. - Test all moveable parts to ensure they do not malfunction or present a hazard. - Check for cracked, split or broken uprights, braces, steps, or rungs. - Locate the Duty Rating sticker to ensure the ladder meets the needs of the worker - Check the stability of the ladder - Check for loose or bent hinge spreaders. - Ladder should be clean, free from grease, oil, wet paint, or any slippery material to prevent slips and falls. - Slip resistant shoes should be - always worn. - Check for protruding screws, rivets, broken metal, or other metal parts that may - cause a cut or scrape during use. | L | Cheyne |
| 2 | Carrying the ladder to the work area. | - Muscle Pull/Strain | H | - Use proper lifting and carrying techniques. Ask for help if the ladder is too heavy. - Bend at the knees and lift the ladder with your leg muscles instead of your back muscles. - Limit turning and twisting motions while carrying the ladder. This may cause a back injury | H | Cheyne |
| 3 | Set up ladder | Back and shoulder sprains and strains Uneven Surface Debris Hazards Slips/Trips/Falls Electrical Hazards | H | - Position ladder close enough to avoid reaching after climbing. Secure legs at the top and bottom. Ensure feet are on firm level ground. - Make sure the ladder is secured at the top and bottom - Position the ladder to face the work activity and not side on. - Make sure the surface that the ladder is to be placed on is even, dry, and constructed of a solid material. - Before and after setup of the ladder, make sure there is no debris above or near the ladder that could pose a potential falling object hazard or slipping hazard. - Place the ladder in a position that is safe. - If a ladder must be placed in front of a door or walkway area make sure, the area is blocked off from pedestrians or a spotter (another employee) is present to guide pedestrians. | H | Cheyne |
| 4 | Climb ladder | Tip over loss of balance | H | - Face ladder, climb slowly, and hold side rails with both hands. - Keep work area clean. and ensure access/egress area is also clear - Be aware of your surroundings. - Don’t stand and work on the top three steps (including a step forming the very top of the stepladder) unless there is a suitable handhold. - Maintain three points of contact at the working position. - Ensure any locking devices are engaged. - NEVER climb on or above the 2nd rung from the top of the ladder or sit on the top of the ladder. - Do not carry heavy materials up the ladder while climbing. - Keep body centred between the side rails to maintain balance | H | Cheyne |
| 5 | Working off ladder | - Reaching out too far - Pinch Points, Strains, and Sprains - Falls Electric shock | H | - Do not allow centre of body to pass side rails of ladder - Think about body positioning; avoid overreaching, hyper extending, location/ position of extremities, are you in the best position for leverage? - face the ladder always using 3 points of contact. Both feet firmly on ladder rung. - No ladder shall be erected within 4 meters of a power line. - Try to avoid work that imposes a side loading, such as side-on drilling. - Never leave objects on top of an unattended ladder. - NEVER ‘walk’ or ‘jog’ the - ladder to another work area. - NEVER stand on or above the 2nd rung from the top of the ladder or sit on the top of the - ladder to perform a job task. - Make sure all objects on the ladder and above the ladder are secure. - Have a spotter present if working around other employees or pedestrians. | H | Cheyne |
| 6 | Descending ladder | Falls from jumping or slipping | H | - Do not rush, use side rails, use all steps, do not jump - Always maintain three points of contact. - Always face the ladder when climbing down. - Keep body centred between the side rails to maintain balance. - If needed, have another employee hold, and secure the base of the ladder before climbing down. - Remove all objects/tools from the top of the ladder before climbing. | H | Cheyne |